Notes about chap6 from RVC v2

LOCALIZATION AND MAPPING

Intro :

Dead reckoning is the estimation of position based on estimated speed, direction

and time of travel with respect to a previous estimate, using elementary geometry.

- Dead reckoning relies on continuous updates of position based on movement estimates.

- It accumulates errors over time due to systematic and random inaccuracies in measurements.

- Historically used for navigation before GPS, it remains relevant for robots operating in environments without GPS.

Landmark map :

A landmark is a visible feature in the environment whose position is known

with respect to some coordinate frame. SEE A MAP AND A NUMBER OF LIGHTHOUSE LANDMARKS :

- This figure demonstrates how landmarks can be used to localize a robot or vessel. It shows two lighthouses (A and C ) and their corresponding positions on a map.

  - By measuring the bearing angles to these landmarks, the robot can triangulate its position.

  - If the wrong landmark is identified (e.g., mistaking B for C ), the estimated position (q ) will be incorrect.

    - Landmarks provide additional information to correct dead-reckoning estimates.

    - Triangulation requires accurate identification of landmarks and precise measurement of bearing angles.

    - Errors in landmark identification or measurement lead to significant localization errors.

RESECTIONING : estimation of position by measuring the bearing angles to known

landmarks

TRIANGULATION : estimation of position by measuring the bearing angles to an unknown point from each  of the landmarks.

Robotis operating tday in environments without GPS face exactly the same

problems as the ancient navigators and, perhaps surprisingly

map. Additional information from observed features in the world is critical to minimizing

a robot’s localization error, but the possibility of data association errors

remains.

It is useful to describe the robot’s estimated position in terms of a probability

density function (PDF) over all possible positions of the robot.

where the magnitude of the function f.x;y/ at any point .x; y/ is the relative likelihood of the robot being at that position

Determining the PDF based on knowledge of how the robot moves and its observations

of the world is a problem in estimation, which we can define as:

» the process of inferring the value of some quantity of interest, x, by processing data

that is in some way dependent on x.

For our robot localization problem, the true and estimated state are vector quantities.

- This figure illustrates the concept of probability density functions (PDFs) to represent the robot's position uncertainty.

    - (a) Low position uncertainty (σ = 1): The robot is confident about its position.

    - (b) High position uncertainty (σ = 20): The robot has significant uncertainty about its location.

    - (c) Multiple hypotheses: The robot believes it could be at one of four possible locations.

  - Localization involves estimating the robot's position with associated uncertainty.

  - PDFs provide a probabilistic representation of the robot's belief about its position.

  - High uncertainty (σ) indicates that the robot is less confident about its location.

Contens  to cover:

6.1 Dead Reckoning Using Odometry

Estimating the robot’s position based on its initial pose, velocity and time.

Some problems: accumulation of errors over time due to sensor noise and wheel slippage.

Inaccurate in environments with significant disturbances.

6.1.1 Modeling the robot

6.1.2 Estimating pose

- Estimates pose from wheel encoders or gyroscopes.

- Pose update: q(k+1)=f(q(k),δ(k),v(k)) .

- Odometry noise: v(k)∼N(0,V) .

- EKF for recursive estimation.

- Uncertainty grows over time.

- confidence ellipses

6.2 Localizing with a Landmark Map

Using known landmarks to refine robot’s position estimate.

How ?

    - Measure bearing angles to land marks or

    - Use triangulation to compute robots position

Some problems :

    - Data association

    - Sensor noise and changes to environment that affects landmark visibily

Sensor measures range and bearing

observation model

Kalman filter updates pose using landmarks observations

reduces uncertainty significantly

data association critical for correct mapping

6.3 Creating a Landmark Map

Construct a map of the environment by detecting and localizing landmarks

We can :

Detect salient features in the environment.

Associate detected features with land marks in the map

then we update the map as the robot explores

some problems:

feature ambiguity

handling occlusions

lighting changes

ensure consistency and completeness of the map

this is useful to build reference maps for future problems.

State vector includes landmark coordinates.

Covariance matrix tracks landmark uncertainties

Function g(.) maps sensor data to world coordinates

WKF extends state vector for new landmarks

Results show evolving confidence ellipses.

6.4 Simultaneous Localization and Mapping

6.5 Pose-Graph SLAM

6.6 SequentialMonte-Carlo Localization

6.7 Rao-Blackwellized SLAM

6.8 Application: Lidar

6.9 Wrapping Up